RISC-V Assembly

EXERCISE # 03

RISCV Assembly Tasks

| Name | Muddassir Ali Siddiqui |
| --- | --- |
| Instructor | Sir Imran & Sir Umer |
| Date | 19th Aug 2025 |

# In-Lab Tasks: (Write your lab task & screenshots here)

## Task 1:

# Question # 01

.data

# # define constant n = 0x7FF (fits in 12-bit imm)

.equ n, 6

.text

j main

fact:                       # int fact(int n)

addi sp, sp, -8         # make stack frame

sw ra, 0(sp)            # save return address

sw s0, 4(sp)            # save s0

mv s0, a0               # move n into s0

bge s0, t0, Recurse     # if (n >= 1) go to Recurse

li a0, 1                # return 1 (base case)

j Epilogue

Recurse:

addi a0, s0, -1         # a0 = n-1

jal ra, fact            # call fact(n-1)

mv t1, a0               # store result of fact(n-1) in x6

mul a0, s0, t1          # a0 = n * fact(n-1)

Epilogue:

lw ra, 0(sp)            # restore return address

lw s0, 4(sp)            # restore s0

addi sp, sp, 8          # pop stack

jr ra                   # return a0

main:

li a0, n                # j = fact(5)

addi t0, x0, 1

jal ra, fact            # call fact

mv s0, a0               # save result in s0

## Task 2:

# Question # 02

.data

# define constant n = 0x7FF (fits in 12-bit imm)

.equ n, 6

.text

jal main

fib:                        # int fib(int n)

addi sp, sp, -12        # make stack frame (need extra space)

sw ra, 0(sp)            # save return address

sw s0, 4(sp)            # save s0

sw s1, 8(sp)            # save s1

mv s0, a0               # s0 = n

beq s0, x0, Case0       # if (n == 0) return 0

addi t0, x0, 1

beq s0, t0, Case1       # if (n == 1) return 1

# ---- Recursive case ----

addi a0, s0, -1         # a0 = n-1

jal ra, fib             # call fib(n-1)

mv s1, a0               # save result of fib(n-1) in s1

addi a0, s0, -2         # a0 = n-2

jal ra, fib             # call fib(n-2)

add a0, s1, a0          # a0 = fib(n-1) + fib(n-2)

j Epilogue

Case0:

li a0, 0                # return 0

j Epilogue

Case1:

li a0, 1                # return 1

Epilogue:

lw ra, 0(sp)            # restore return address

lw s0, 4(sp)            # restore s0

lw s1, 8(sp)            # restore s1

addi sp, sp, 12         # pop stack

jr ra                   # return a0

main:

li a0, n                # j = fib(6)

jal ra, fib             # call fib

mv s0, a0               # save result in s0