RISC-V Assembly

EXERCISE # 01

RISCV Assembly Tasks

| Name | Muddassir Ali Siddiqui |
| --- | --- |
| Instructor | Sir Imran & Sir Umer |
| Date | 19th Aug 2025 |

# In-Lab Tasks: (Write your lab task & screenshots here)

## Task 1:

# Question # 01

.data

.equ m1, -76

.equ m2, 5

.globl main

.text

# int multiply(int a0, int a1) -> result in a2

multiply:

addi sp, sp, -16       # make stack frame

sw ra, 12(sp)          # save return address

sw a0, 8(sp)           # save a0

sw a1, 4(sp)           # save a1

li a2, 0               # result = 0

li t0, 0               # sign = 0

# Check sign of a0

bltz a0, negA

skipA:

j checkB

negA:

sub a0, x0, a0         # make positive

xori t0, t0, 1         # flip sign

checkB:

bltz a1, negB

skipB:

j multiply_loop

negB:

sub a1, x0, a1

xori t0, t0, 1         # flip sign

# Repeated addition loop

multiply_loop:

beq a1, x0, done       # while (a1 != 0)

add a2, a2, a0         # result += a0

addi a1, a1, -1

j multiply_loop

done:

beq t0, x0, end_sign   # if sign == 0 skip

sub a2, x0, a2         # apply negative

end_sign:

lw ra, 12(sp)          # restore ra

lw a0, 8(sp)           # restore a0 (optional)

lw a1, 4(sp)           # restore a1 (optional)

addi sp, sp, 16        # restore stack

ret

main:

# Example inputs: a0 = -6, a1 = 5

li a0, m1          # multiplicand

li a1, m2          # multiplier

jal multiply

# Now result is in a2

# (Here you can put ecall to print if running on RARS/Venus)

mv s0, a0

mv s1, a1

mv a1, a2

li a0, 1

ecall

li a0, 10          # exit

ecall

## Task 2:

# Question # 02

.data

# Change values here for testing

.equ m1, -740     # dividend

.equ m2, 5       # divisor

.globl main

.text

# int divide(int a0, int a1) -> quotient in a2, remainder in a3

divide:

addi sp, sp, -16       # make stack frame

sw ra, 12(sp)          # save return address

sw a0, 8(sp)           # save dividend

sw a1, 4(sp)           # save divisor

li a2, 0               # quotient = 0

mv a3, a0              # remainder starts as dividend

li t0, 0               # sign flag for quotient

li t1, 0               # sign flag for remainder (same as dividend)

# Check sign of dividend (a0)

bltz a0, negDividend

skipDividend:

j checkDivisor

negDividend:

sub a0, x0, a0         # make dividend positive

sub a3, x0, a3         # remainder also positive

xori t0, t0, 1         # flip quotient sign

xori t1, t1, 1         # remainder has same sign as original dividend

checkDivisor:

bltz a1, negDivisor

skipDivisor:

j division_loop

negDivisor:

sub a1, x0, a1         # make divisor positive

xori t0, t0, 1         # flip quotient sign

# Division loop: while (dividend >= divisor)

division_loop:

blt a0, a1, done       # if dividend < divisor -> stop

sub a0, a0, a1         # dividend -= divisor

addi a2, a2, 1         # quotient++

j division_loop

done:

mv a3, a0              # remainder = leftover dividend

# Apply sign to quotient

beq t0, x0, skipQsign

sub a2, x0, a2

skipQsign:

# Apply sign to remainder (same sign as original dividend)

beq t1, x0, skipRsign

sub a3, x0, a3

skipRsign:

lw ra, 12(sp)          # restore ra

lw a0, 8(sp)           # restore a0 (optional)

lw a1, 4(sp)           # restore a1 (optional)

addi sp, sp, 16        # restore stack

ret

main:

li a0, m1          # dividend

li a1, m2          # divisor

jal divide

# Now quotient in a2, remainder in a3

# Print quotient

mv a1, a2

li a0, 1

ecall

# Print new line

addi a1, x0, '\n'

addi a0, x0, 11  # Print newline

ecall

# Print remainder

mv a1, a3

li a0, 1

ecall

li a0, 10          # exit

ecall