RISC-V Assembly

Lab Manual # 12

Introduction to RISC-V Assembly & Custom Processor Design

| Name | Muddassir Ali Siddiqui |
| --- | --- |
| Instructor | Sir Imran & Sir Umer |
| Date | 13th Aug 2025 |

# In-Lab Tasks: (Write your lab task & screenshots here)

Part 1:

## Task1

## Task 2

### Unique Assembler Directives

| Directive | Purpose |
| --- | --- |
| .file "hello.c" | Records the source file name for debugging. |
| .option nopic | Disables Position Independent Code (absolute addresses instead). |
| .attribute arch, ... | Specifies target RISC-V architecture and extensions. |
| .attribute unaligned_access, 0 | Declares unaligned memory access is not supported. |
| .attribute stack_align, 16 | Ensures stack is always 16-byte aligned (ABI requirement). |
| .text | Marks start of the text (code) section. |
| .section .rodata | Switches to read-only data section for constants/strings. |
| .align 3 | Aligns data to an 8-byte boundary (2³ = 8). |
| .string "Muddassir_Ali_Siddiqui" | Stores a null-terminated string constant. |
| .align 2 | Aligns following code/data to 4-byte boundary (2² = 4). |
| .globl main | Makes main visible to linker as a global symbol. |
| .type main, @function | Declares main as a function symbol. |
| .size main, .-main | Defines the size of main function. |
| .ident "GCC: ..." | Embeds compiler identification string. |
| .section .note.GNU-stack,"",@progbits | Marks stack as non-executable for security. |

### Unique Base Instructions

addi

lui

ld

sd

### Unique Labels

.LCO:

main:

### Unique Pseudo instruction and their corresponding base instructions.

call printf 		auipc ra, %pcrel_hi(printf); jalr ra, %pcrel_lo(printf)(ra)

li a5, 0			addi a5, x0, 0

mv a0, a5		addi a0, a5, 0

jr ra			jalr x0, 0(ra)

## Task 3

Part 2:

## Task 1:

## Task 2:

## Task 3:

## Task 4:

## Task 5:

# Critical Analysis: (Write you critical analysis / conclusion here)

In this lab, we designed and implemented the fundamental modules of a simple processor, including a 16-bit ALU, a 16-bit Register File with four registers, an 8-bit Instruction Memory, and a Program Counter capable of addressing up to 4 instructions. All components were parameterized to allow flexibility in width and depth. After integration at the top level, the processor was initialized with fib_im.mem and fib_rf.mem, successfully executing the instructions. This lab provided practical insight into how parameterized hardware blocks interact to form a working microprocessor.